# Thoughts on the European timetable profile for rail

Input documents:

- Fares and Timetable Use-Cases description and mapping for PT and Rail

- Telematics TSI technical document B.17

- Examples/tap_tsi/Timetable

Goal:

- Define a European timetable profile for rail to be used in the rail domain for the exchange of timetable data

TSDUPD:

- Common understanding of the split between the elements:

  - StopPlace – defining the physical infrastructure of a station

  - ScheduledStopPlace – defining the logical infrastructure to be used for timetable data provision

- Common understanding of centrally managed elements

  - Organisations

  - Location codes “EU-Id”

  - Code lists

SKDUPD:

Questions:

- Topographic place necessary?

  - Who is in charge of them?

  - Re-using of EU wide available data, e.g. NUTS?

- How the identifiers @id should be used in the file:

  - Using the EU-ID as numerical part of the id?

  - Re-using the existing retail ID

  - Shall we use the object types for the distinction between the different objects e.g. ScheduledStopPoint:<EU-id>… , StopPlace:<EU-id>….

- ScheduledStopPoint/tariffZones – is not a tariff zone, its an ID for the reservation system, to be replaced by keyList for reservation code including the operator / reservation system

# Elements used in SKDUPD:

## 1. Document Level

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| PublicationDelivery | M | 1 | Root element of a NeTEx publication | Must be present |
| PublicationDelivery/@version | M | 1 | NeTEx publication version | 1.0 |
| PublicationDelivery/@xsi:schemaLocation | M | 1 | Schema reference | NeTEx_publication.xsd |

## 2. Publication Metadata

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| PublicationDelivery/PublicationTimestamp | M | 1 | Creation timestamp | ISO 8601 |
| PublicationDelivery/ParticipantRef | M | 1 | Data provider identifier | Railway undertaking identified by the organisation code |
| PublicationDelivery/Description | O | 1 | Human-readable description | Free text |
| PublicationDelivery/Version | M | 1 | Publication version | TODO |

## 3. Data Container

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| PublicationDelivery/dataObjects | M | 1 | Container for exchanged data | Mandatory wrapper |
| PublicationDelivery/dataObjects/CompositeFrame | M | 1 | Container for exchanged data | Mandatory wrapper |
| codespaces | ??? |  |  |  |
| dataObjects/frames/ServiceCalendarFrame | M | 1..R | Service calendar for timetable | Service calendar |
| dataObjects/frames/ServiceFrame | M | 1..R | Service frame for timetable | Scheduled stop points + distances between them |
| dataObjects/frames/TimetableFrame | M | 1..R | Timetable data frame | Timetable dataset |

## 4. Service Calendar Frame

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| ServiceCalendarFrame/@id | M | 1 | Frame identifier | Unique – composed by ??? |
| ServiceCalendarFrame /@version | M | 1 | Frame version | Numeric default any? |
| ServiceCalendarFrame/operatingPeriods | M | 1 | Container for operating periods |  |
| ServiceCalendarFrame/ operatingPeriods/UicOperatingPeriod | M | 1..n | Operating period | Operating period |
| UicOperatingPeriod /@id | M | 1 | Frame identifier | Unique – composed by fromdate_todate_validbits |
| UicOperatingPeriod /@version | M | 1 | Frame version | Numeric default any? |
| ServiceCalendarFrame / FromDate | M | 1 | Validity period | Start date |
| ServiceCalendarFrame / ToDate | M | 1 | Validity period | End date |
| ServiceCalendarFrame/ValidDayBits | M | 1 | Validity period | Validdays bitmask |

## 5. Service Frame

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| ServiceFrame/@id | M | 1 | Frame identifier | Unique |
| ServiceFrame/@version | M | 1 | Frame version | Numeric default any? |
| ServiceFrame /Name | O | 1 | Frame name | Informative |
| ServiceFrame /scheduledStopPoints | M | 1 | Container for scheduledStop points |  |
| ServiceFrame /scheduledStopPoints/scheduledStopPoint | M | 1...n | Scheduled stop point | Referenced in the calls |
| scheduledStopPoint/@id | M | 1 | Frame identifier | EU-ID |
| scheduledStopPoint /@version | M | 1 | Frame version | Numeric default any? |
| scheduledStopPoint/Name | M | 1..n |  | Unique per lang |
| scheduledStopPoint/Name/@lang | O | 1 | language |  |
| scheduledStopPoint/ShortName | O | 1 | shortname |  |
| scheduledStopPoint/StopType | M | 1 | Type of the station | railStation |
| scheduledStopPoint/CountryRef | O | 1 | Country of the stop point |  |

## 6. Service links

## Is this still necessary??

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| ServiceFrame/serviceLinks | O | 1 |  | Used for the provision of distances |
| ServiceLinks/ServiceLink | M | 1..n | Service links |  |
| ServiceLink/@id | M | 1 | Frame identifier | Unique is combined from station to station |
| ServiceLink/@version | M | 1 | Frame version | Numeric default any? |
| ServiceLink /Distance | M | 1 | Distance | Units to be checked (meters / kilometers) |
| ServiceLink /FromPointRef | M | 1 | Starting point |  |
| ServiceLink /ToPointRef | M | 1 | Destination |  |

## 7. Timetable Frame

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| TimetableFrame/@id | M | 1 | Frame identifier | Unique |
| TimetableFrame/@version | M | 1 | Frame version | Numeric |
| TimetableFrame/Name | O | 1 | Frame name | Informative |
| TimetableFrame/vehicleModes | M | 1 |  | rail |

## 8. Service Journeys

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| TimetableFrame/vehicleJourneys | M | 1 | Container for journeys | Mandatory |
| vehicleJourneys/ServiceJourney | M | 1..n | Train service journey | Core business object |
| ServiceJourney/@id | M | 1 | Journey identifier | Operator-specific. Using uniqueTrainID??? |
| ServiceJourney/Name | O | 1 | Journey name | Train label |
| ServiceJourney/PrivateCode | M | 1 | Operational train number |  |
| ServiceJourney/OperatorRef | M | 1 | Operator reference | Railway company |
| ServiceJourney/dayTypes/DayTypeRef | M | 1 | Operating period | Mandatory |
| TypeOfProductCategoryRef |  |  |  |  |
| TypeOfServiceRef |  |  |  |  |
| DepartureTime |  |  |  |  |
| trainNumbers/TrainNumberRef |  |  |  |  |

## 9. parts / JourneyPart

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| ServiceJourney/parts | M | 1 | Parts of a vehicle journey | Mandatory |
| parts/JourneyPart | M | 1..n | Stop call | Ordered sequence |
| To be completed |  |  |  |  |

## 10. Calling Points / Timing

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| ServiceJourney/calls | M | 1 | Sequence of stops | Mandatory |
| calls/Call | M | 1..n | Stop call | Ordered sequence |
| Call/@id | M |  |  |  |
| Call/@version | M |  |  | Any? |
| Call/@order | M | 1 | Order of stops |  |
| Call/SCheduledStopPointRef | M | 1 | Scheduled stop | Station reference |
| Call/Arrival/Time | O | 1 | Arrival time | ISO time |
| Time/ForAlighting | O | 1 | Alighting allowed |  |
| Call/Departure/Time | O | 1 | Departure time | ISO time |
| Time/ForBoarding | O | 1 | Boarding allowed |  |
| Call/Departure/DayOffset | O | 1 | Day offset | Used for trains in case the stop time crossed midnight |
| Call/PlatformRef | O | 1 | Platform / track | If known |

## 11. Validity Conditions

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| TimetableFrame/validityConditions | M | 1 | Validity container | Mandatory |
| validityConditions/UicOperatingPeriod | M | 1..n | Operating period |  |
| UicOperatingPeriod/FromDate | M | 1 | Start date | ISO date |
| UicOperatingPeriod/ToDate | M | 1 | End date | ISO date |
| UicOperatingPeriod/ValidDayBits | M | 1 | Days of operation | Bit mask |

## 12. TrainNumbers

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| TimetableFrame/trainNumbers | M | 1 | Train numbers container | Mandatory |
| trainNumbers/TrainNumber | M | 1..n | trainNumbers |  |
| trainNumber /@id | M | 1 | Start date | UniquetrainID??? |
| trainNumber /@version | M | 1 | Frame version | Numeric default any? |
| trainNumber/ForAdvertisement | O | 1 | Used for passenger information |  |
| trainNumber/ForProduction | O | 1 | Operation train number | UniquetrainID??? |

## 13. journeyMeetings

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| TimetableFrame/journeyMeetings | O | 1 | Journeymeetings container |  |
| journeyMeetings/JourneyMeeting | M | 1..n | journeyMeeting events |  |
| JourneyMeeting/@id | M | 1 | Start date | vehicleJourneyFrom+vehicleJourneyTo??? + |
| JourneyMeeting/@version | M | 1 | Frame version | Numeric default any? |
| JourneyMeeting/FromJourneyRef | M | 1 |  | From Journey |
| JourneyMeeting/ToJourneyRef | M | 1 |  | To journey |
| JourneyMeeting/Reason | M | 1 |  | Reason for such a meeting |

## 14. journeyInterchanges

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| TimetableFrame/journeyInterchanges | O | 1 | Journey interchanges container |  |
| journeyInterchanges/ServiceJourneyInterchange | M | 1..n | ServiceJourneyInterchanges |  |
| ServiceJourneyInterchange/@id | M | 1 | Start date | UniquetrainID??? |
| trainNumbers /@version | M | 1 | Frame version | Numeric default any? |
| trainNumbers/ForAdvertisement | M | 1 | Days of operation | Bit mask |
| trainNumbers/ForProduction |  |  |  |  |

## 15. Elements Explicitly Not Used in This Profile

| Element / Area | Usage | Comment |
| --- | --- | --- |
| Fare elements | X | Not part of TSDUPD |
| Passenger accessibility | X | Out of scope |
| Alternative modes | X | Rail only |
| GroupOfLines / Routes | X | Not used in TAP-TSDUPD |
| Network topology objects | X | Reference only |

# Elements used in TSDUPD:

## 16. Topographic places

## Is this still necessary??

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| SiteFrame/topographicPlaces | O | 1 | Topographic places container |  |
| topographicPlaces/ TopographicPlace | M | 1..n | Topographic place |  |
| TopographicPlace/@id | M | 1 |  | ??? |
| TopographicPlace/@version | M | 1 | Frame version | Numeric default any? |
| Descriptor/Name | M | 1 | Name of the topographic place |  |

## 17. Stop places

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| SiteFrame/stopPlaces | O | 1 | Journey interchanges container |  |
| stopPlaces/StopPlace | M | 1..n | ServiceJourneyInterchanges |  |
| StopPlace /@id | M | 1 | Unique id | EU-ID??? |
| StopPlace /@version | M | 1 | Frame version | Numeric default any? |
| StopPlace /TransportMode | M | 1 | rail |  |
| StopPlace / StopPlaceType | M | 1 | railStation |  |

General remark: Maybe the stop places shall be aligned with the requirements from PRM TSI concerning the accessibility parameters for any station.

## 18. Stop assignments

| Element Path | Usage | Cardinality | Description | Example / Comment |
| --- | --- | --- | --- | --- |
| ServoeFrame/stopAssignments | O | 1 | Stop assignments container | Assignment between the stop place and the scheduled stop place |
| stopAssignments/PassengerStopAssignment | M | 1..n | PassengerStopAssignment | Why not TrainStopAssginment? |
| PassengerStopAssignment /@id | M | 1 | Unique id | EU-ID??? |
| PassengerStopAssignment /@version | M | 1 | Frame version | Numeric default any? |
| PassengerStopAssignment/Name | O | 1 |  |  |
| PassengerStopAssignment/Description | O | 1 |  |  |
| PassengerStopAssignment/ ScheduledStopPointRef | M | 1 | Reference to the scheduled sto point | EU-ID??? |
| PassengerStopAssignment/ StopPlaceRef | M | 1 | Reference to the stopplace | EU-ID??? |